INTRODUCTION

As of 2018, 14% and 8% of global oil and gas, respectively, are consumed as feedstock for the chemical industry [IEA 2018]. Out of all the chemicals, ammonia, methanol, ethylene, and propylene are the major building blocks of the chemical industry which are further used as raw materials for the production of valuable products. Ammonia is predominantly produced via the Haber-Bosch process using natural gas as feedstock and is responsible for one-fourth of the total fossil-fuel consumption in the chemical industry [Carlo D’Angelo et al. 2021]. Around 90% of the ammonia produced globally is used for the production of fertilizers. Ammonia alone is responsible for the release of approximately 550 Mt CO2eq which accounts for 1.5% of global carbon emissions [IEA 2013]. The major source of these emissions is the hydrogen required for ammonia synthesis which is produced via Steam Methane Reforming (SMR) [Carlo D’Angelo et al. 2021]. Similarly, methanol accounts for around 0.3% of global carbon emissions [IEA 2013]. Natural gas or coal is used for the production of syngas which is then converted to methanol via the water-gas shift reaction. This rather simple chemical compound is used for the production of valuable products such as formic acid, acetic acid, dimethyl ether, methylamine, etc. [Mohammed El-Zeftawy 1995].

Ethylene is majorly produced via high temperature (750 – 900 °C) steam cracking of hydrocarbons [Kirk Othmer]. It is the world’s largest chemical produced on a volume basis, with a global capacity of 214 Mt (2021) annually [GlobalData]. Steam cracking of hydrocarbons for the production of ethylene yields various by-products such as propylene, butadiene, benzene, etc. Both ethylene and propylene are regarded as the most important olefins as they are used for the production of plastics and resins [Kirk Othmer]. Ethylene and propylene together are responsible for 1.2% of global carbon emissions [IEA 2013]. Therefore, owing to climate change and the targets set by the Paris Agreement, it is important to find alternative ways for the production of these four chemicals which currently account for 3% of the total global carbon emissions [González‑Garay et al. 2021].

In Europe, since July 2021, natural gas which is used as a chemical feedstock for the production of ammonia and methanol has seen a sharp increase in prices. The major reasons for this trend are the impact of COVID, skyrocketing inflation, and geopolitical scenarios that exist today. In the past year, monthly average natural gas prices have increased from 560$ t-1 in July 2021 to 2300$ t-1 as of July 2022 [World Bank]. Similarly, the price of grid electricity has almost quadrupled [Ember]. For Europe, the annual demand is around 19 Mt for ammonia [IEA 2020] and 5 Mt for methanol [Hung Vo et al. 2021, MMSA 2020]. Therefore, increasing natural gas prices and global carbon emissions provide us with a great opportunity to look for alternative routes of production for chemicals and invest in future technologies. In this work, we focus on green routes for the production of ammonia and methanol and compare them with conventional technologies in the present state of affairs.