# Supplemental experimental procedures

S1. Economic assessment

S1.1. Ammonia. The inventories involving the material balances and energy flows for both fossil and green ammonia are extracted from the work done by D’Angelo et al.1. In the case of fossil ammonia, every ton of ammonia produced consumes 0.53 tons of natural gas and 940 kWh of electricity1. The prices of feedstock and utilities other than natural gas, electricity, and hydrogen are extracted from D’Angelo et al.1. These prices (Table S1) are adjusted for inflation using the Chemical Engineering Plant Cost Index – CEPCI (from Table S2) when considering data for a different year2. The monthly average natural gas spot prices (i.e., prices for new contracts at the Title Transfer Facility (TTF) trading point) from World Bank3 (Fig. 1) and electricity prices from Ember4 (Fig. S1) are used for all our calculations. Further, the levelized cost for on-shore wind and solar electricity and their corresponding capacity factors are extracted from the International Renewable Energy Agency (IRENA) for 2019 to 2021 (except 2022)5–7. Using these values, the levelized cost of hydrogen (LCOH) is calculated following the assumption and equations provided in the work carried out by D’Angelo et al.1. Due to the lack of availability of similar data for the year 2022, the levelized cost of hydrogen for the year 2021 is adopted and adjusted for inflation using the CEPCI (Table S2)2. The levelized cost of electricity and hydrogen for the corresponding years are shown in Table S3 and Table S4, respectively. A detailed overview of the LCOH calculations is demonstrated in section S1.3.

S1.2. Methanol. The inventories for fossil and green methanol are similarly extracted from the work by González-Garay et al.8. Every ton of fossil methanol consumes 0.65 tons of natural gas and 74 kWh of electricity8. The prices of natural gas (Fig. 1), electricity (Fig. S1), and hydrogen (Table S4) are substituted by those mentioned earlier in the case of ammonia, and all the other costs are adjusted for inflation (Table S5). The price of CO2 from Direct Air Capture (DAC) is extracted from Keith et al.9  and recalculated with the new natural gas prices (Fig. 1). For every ton of CO2 captured using the DAC technology, 0.11 tons of natural gas is required9.

S1.3. Levelized cost of hydrogen (LCOH). The parameters used to calculate the LCOH for all on-shore wind and solar-based scenarios are shown in Table S6. A Proton Exchange Membrane (PEM) electrolyzer with a stack lifetime of 60000 hours is considered for the analysis1. The equations used to calculate the LCOH for hydrogen production from water electrolysis using on-shore wind and solar electricity are as follows1:

Life span = stack lifetime × 1008760 × capacity factor

Annual H2 production = nominal H2flow per stack × stack lifetimelife span

CAPEX = i= -10nominal input power × capital cost × 0.5(1 + interest rate)i

H2 produced = i= 1life spanannual H2production × H2 production capacity(1 + interest rate)i

with H2 production capacity= 0.5 i=11.0 i>1

Electricity cost = power demand electrolyzer × LCOE × H2 produced

Other costs = i= 1life spannominal input power × capital cost × 0.04(1 + interest rate)i

Plant decommissioning = i= 1life spannominal input power × capital cost × 0.1(1 + interest rate)i

Equipment replacement costs = i= 1floor(life span7)nominal input power × capital cost × 0.15(1 + interest rate)i

OPEX = electricity cost + other costs + plant decommissioning + equipment replacement costs

LCOH = CAPEX + OPEXH2 produced

Fig. S1. Evolution of electricity prices in Europe4.

Fig. S2. Breakdown of production costs for fossil and green ammonia (a) and methanol (b). Error bars display the production cost considering the lower and upper bound for the levelized cost of hydrogen.

Table S1. Cost parameters used in the economic calculations for ammonia1. All costs are presented in USD2019.

| Flow | Average | Low | High |
| --- | --- | --- | --- |
| Water [USD t-1] | 0.18 | - | - |
| Cooling water [USD GJ-1] | 0.38 | - | - |
| CO2 capture and storage [USD t-1] | 15.84 | - | - |
| CO2 emissions [USD t-1] | 30.00 | - | - |
| Nitrogen [USD t-1] | 91.06 | - | - |

Table S2. CEPCI values from 2019 to 20222.

| Year | CEPCI |
| --- | --- |
| 2019 | 607.5 |
| 2020 | 596.2 |
| 2021 | 708.0 |
| 2022 (average till September) | 817.5 |

Table S3. Levelized cost of electricity in USD MWh-1 and capacity factors (CF) in %5–7.

|  | 2019 |  |  |  | 2020 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average | Low | High | CF | Average | Low | High | CF |
| Wind - on shore | 67 | 37 | 96 | 35.6 | 45 | 35 | 65 | 36.0 |
| Solar PV | 68 | 52 | 190 | 17.5 | 57 | 39 | 163 | 16.1 |

|  | 2021 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Average | Low | High | CF |
| Wind - on shore | 42 | 26 | 59 | 39.0 |
| Solar PV | 48 | 29 | 120 | 17.2 |

Table S4. Levelized cost of hydrogen in USD t-1. The methodology used to calculate the LCOH values is described in section 1.3.

|  | 2019 |  |  | 2020 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average | Low | High | Average | Low | High |
| Wind - on shore | 6882 | 5232 | 8477 | 5636 | 5086 | 6736 |
| Solar PV | 9217 | 8337 | 15927 | 9047 | 8057 | 14877 |

|  | 2021 |  |  | 2022 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average | Low | High | Average | Low | High |
| Wind - on shore | 5379 | 4499 | 6314 | 6211 | 5195 | 7290 |
| Solar PV | 8212 | 7167 | 12172 | 9482 | 8276 | 14055 |

Table S5. Cost parameters used in the economic calculations for methanol8. All costs are presented in USD2019.

| Flow | Average | Low | High |
| --- | --- | --- | --- |
| Steam [USD t-1] | 15.60 | - | - |
| Cooling water [USD m-3] | 0.03 | - | - |
| Catalyst [USD t-1] | 136.38 | - | - |
| Wastewater treatment [USD m-3] | 1.64 | - | - |
| Carbon dioxide from DAC [USD t-1] | 137.35* | 67.85* | 206.50* |

*These prices do not include the natural gas cost required to capture carbon dioxide. One ton of carbon dioxide requires 0.11 tons of natural gas9.

Table S6. Parameters for the PEM electrolyzer1. All values are in reference to the year 2019.

| Component | Value |
| --- | --- |
| Nominal input power [kW] | 5000 |
| Nominal hydrogen flow [kg hr-1] | 89.9 |
| Stack lifetime [hrs] | 60000 |
| Power demand electrolyzer [kWh Nm3H2-1] | 5 |
| Capital cost [USD kW-1] | 1123.6 |
| Interest rate [%] | 6.4 |
| Decommissioning cost (fraction of depreciable capital) [%] | 10 |
| Equipment replacement cost (fraction of initial CAPEX) [%] | 4 |
| Construction period [years] | 2 |
| Start-up period [years] | 1 |
| Start-capacity [%] | 50 |

Note S1. Natural gas TTF spot prices.

For our calculations, we use Natural Gas TTF spot prices (monthly average). TTF stands for Title Transfer Facility and is a virtual hub situated in the Netherlands where contracts are made based on both spot and future prices. In the past years, the TTF index has gained significant importance. It is now by far the most liquid, most traded hub, with the greatest number of participants of all the European traded hubs. Indeed, TTF has become the mature risk management hub, not just for the Netherlands but for all of Europe and beyond. The metric that shows how TTF has grown almost exponentially in recent years is the total volumes traded. TTF’s traded volumes now account for 79% of the total European traded gas markets. TTF has become the pricing benchmark for North-West Europe, although several other countries across Europe will also use TTF to price contracts. TTF is also used to price some LNG cargoes destined for Europe.

Therefore, if a company wants to make new contracts for natural gas, it will use the TTF index for such deals. Most of these companies make contracts based on future prices. These prices are mutually decided depending on the market conditions. However, if a company needs natural gas at the spot, i.e., immediately, they will be charged the spot prices according to the TTF index.

Even though not all of Europe uses the TTF index, just seeing the trading volumes and share of the European market, it is a safe bet to use the TTF index for our analysis. Also, the spot prices might be higher than what the companies agree to in a contract based on the future. But such data is hard to find or predict, and therefore for this study, we use the spot prices.

Note S2. Ammonia model validation.

The production cost of ammonia was calculated using our model and compared with the data extracted from ICIS10 (Fig. S3). ICIS has published the production cost of ammonia for certain dates from October 2021 to October 2022 using the natural gas TTF spot prices. As shown in Fig. S3, our model is in good agreement with the values from ICIS except at the peak. This divergence can be explained by understanding the prices used by ICIS. ICIS calculates the peak on 26th August 2022, where the natural gas prices were around 4,519 USD t-1, however we use monthly average prices in our analysis and for that reason the natural gas price we use at the peak (August 2022) is around 3,130 USD t-1.

Fig. S3. Ammonia model validation10.

Note S3. Methanol model validation.

Market prices for methanol in Europe are extracted from Methanex Corporation11 (January 2015 to September 2022) and Methanol Marketing Services Asia - MMSA12 (October 2019 to September 2022). MMSA is a consultancy service in Asia that does not provide sufficient information about the data sources. Methanex Corporation, on the other hand, is the world’s largest producer and supplier of methanol to major international markets. As shown in Fig. S4, the data from both Methanex and MMSA are quite similar. Methanex has production sites in countries like Trinidad, New Zealand, Egypt, Canada, the United States, and Chile.

When comparing the cost of production calculated from our model and the market price of methanol, we see that from January 2015 to January 2021, the production costs harmonize well with the market prices of methanol. However, when the natural gas prices start becoming volatile, our model predicts production costs that are way higher than the market price of methanol in Europe. The reasons for this ambiguity are the following:

- Methanex and MMSA report the market prices of methanol. These prices are not the same as the cost of production in Europe. Rather, the methanol is imported and sold from the port of Rotterdam.

- Methanex has production sites in countries that are not affected by the current volatility of natural gas prices and hence produce methanol at a much lower price. When this methanol is exported to Europe, the market price is expected to be lower than the cost of production in Europe where the natural gas prices are the most impacted.

- Our model uses monthly average spot prices of natural gas TTF. One shortcoming of using these prices is that companies in theory will not buy natural gas at these prices. Rather, they depend on contracts. However, the price of natural gas agreed upon by different stakeholders will depend on the natural gas TTF index. Therefore, this analysis does not mean that companies are producing methanol at the prices shown in Fig. S4, it rather shows that it is the right time to invest in alternative technologies so that Europe does not lose its industrial competitiveness.

Fig. S4. Methanol cost of production (model) vs market price (literature)11,12.

References

1.	D’Angelo, S.C., Cobo, S., Tulus, V., Nabera, A., Martín, A.J., Pérez-Ramírez, J., and Guillén-Gosálbez, G. (2021). gAmmonia - main paper. ACS Sustain Chem Eng 9, 9740–9749. 10.1021/acssuschemeng.1c01915.

2.	Charles Maxwell (2022). Cost Indices. Towering Skills.

3.	World Bank (2023). Monthly Prices, Commodity Markets. World Bank.

4.	Ember (2022). European wholesale electricity price data. Ember.

5.	International Renewable Energy Agency (2020). Renewable Power Generation Costs in 2019.

6.	International Renewable Energy Agency (2021). Renewable Power Generation Costs in 2020.

7.	International Renewable Energy Agency (2022). Renewable Power Generation Costs in 2021.

8.	González-Garay, A., Frei, M.S., Al-Qahtani, A., Mondelli, C., Guillén-Gosálbez, G., and Pérez-Ramírez, J. (2019). Plant-to-planet analysis of CO2-based methanol processes. Energy Environ Sci 12, 3425–3436. 10.1039/c9ee01673b.

9.	Keith, D.W., Holmes, G., st. Angelo, D., and Heidel, K. (2018). Carbon dioxide - DAC. Joule 2, 1573–1594. 10.1016/j.joule.2018.05.006.

10.	Deepika Thapliyal (2022). INSIGHT: Crashing gas spurs fertilizer ramp up, petchems still shuttered. Independent Commodity Intelligence Services (ICIS).

11.	Methanex Corporation (2022). Historical Methanex Posted Price. Methanex Corporation.

12.	Methanol Marketing Services Asia (MMSA) (2022). Global Methanol Pricing Comparison. Methanol Market Services Asia.