# Electronic Supplementary Information

# ‘Energy crisis in Europe revamps the economic competitiveness of green chemicals’

## Production routes

## Ammonia

The fossil and green ammonia routes are based on D’Angelo et al.1. For fossil ammonia, natural gas and water are mixed and heated to 858 °C and fed to a SMR unit where natural gas is converted to carbon monoxide and hydrogen (syngas). The output from the SMR is then fed into an autothermal reformer (ATR) along with air (78% nitrogen, 21% oxygen, and 1% argon). The oxygen in the air reacts with the residual methane and increases the temperature to 1,200 °C inside the ATR. The output from the ATR is cooled to 400 °C and sent to a series of water-gas-shift (WGS) reactors to convert carbon monoxide to CO2. The first one operates at a high temperature (260 °C), i.e., high-temperature water-gas-shift (HTWGS) reactor. The output stream from the HTWGS is fed to a low-temperature water-gas-shift (LTWGS) reactor, which operates at 50 °C. The output stream from the WGS reactors is rich in CO2, which needs to be removed. This is done in an absorption column using a lean solvent stream with a composition of 90% water and 10% monoethanolamine (MEA) where 99.7% of the input CO2 is absorbed and removed in the liquid stream. The residual carbon monoxide and CO2 are eliminated to avoid poisoning the catalyst in the Haber-Bosch process2. Accordingly, a methanation unit converts the residual carbon oxides to methane. In the Haber-Bosch process, the CO2 and carbon monoxide free stream are then compressed to 196 bar, mixed with the recycle stream, and sent to a flash separator. This stream is then sent to a series of three reactors where nitrogen and hydrogen react to produce ammonia. As mentioned previously, the outlet stream from these three reactors is cooled and mixed with the fresh feed and sent to a flash unit. The liquid stream from this flash is rich in ammonia, which is further purified with a pair of flash separators to meet industrial standards (99.6%). For the production of ammonia using green hydrogen (electrolytic hydrogen using wind or solar electricity) and blue hydrogen (SMR coupled with CCS), nitrogen from cryogenic air separation and hydrogen is mixed in the ratio 1:3 and sent to the Haber-Bosch process described earlier. A Proton Exchange Membrane (PEM) electrolyser with a stack lifetime of 60,000 hours is considered for producing green hydrogen via water electrolysis. Figure S1 shows a basic visual representation of the ammonia production process.

### Methanol

The fossil and green production routes for methanol are based on González-Garay et al.3. Fossil methanol is produced from syngas from natural gas via SMR. Syngas is then compressed to 110 bar, mixed with the recycle streams and then heated to 150 °C before being fed to a plug-flow reactor where syngas is converted to methanol following the kinetic model in Bussche and Froment4. The output stream from the reactor is cooled to 38 °C and sent to a flash separator. A small portion of the gas stream from the flash separator is purged whereas the rest is pressurized to 110 bar and recycled. The liquid stream from the flash separator is expanded to 2 bar and sent to a second flash unit, where the gas stream is again pressurized to 110 bar and recycled, and the liquid stream is sent to a distillation column. The distillation column separates liquid methanol (at the condenser) with a purity of 99.9% and water (at the bottom). To produce methanol using green and blue hydrogen, CO2 from DAC and hydrogen is mixed with the recycle streams, compressed to 50 bar, and heated to 180-280 °C. This stream is then sent to a reactor where, the reverse water-gas-shift (RWGS) reaction takes place, converting CO2 to carbon monoxide, which is consequently hydrogenated to methanol. Similar to fossil methanol, the outlet from the reactor is sent to a series of two flash separators where the gas stream from the first flash is recycled (with a purge). The liquid stream from the second flash unit is sent to a distillation column which recovers methanol at a purity of 99.9%. The CO2 fed to the reactor is obtained via the DAC technology adapted from the work by Keith et al.5. Here, aqueous potassium hydroxide (KOH) is used as a sorbent and a caustic calcium cycle is implemented for the regeneration of KOH. However, energy from natural gas is required for the calciner in the recovery cycle. Figure S2 shows the graphical representation of the methanol production process.

## Economic assessment

We used data from process simulations to determine the energy and material inputs and outputs as well as the necessary infrastructure associated with each production route. We then estimate the operational and capital expenditures (OPEX and CAPEX) for a reference year (2019), adjusting afterwards the results for other years using the Chemical Engineering Plant Cost Index (CEPCI) to account for inflation. For the OPEX calculations, the natural gas and electricity prices were not adjusted for inflation but instead we consider its fluctuating price at any given point in time (monthly average) based on historical market values. The hydrogen cost is considered as OPEX and estimated from the renewable electricity price and the electrolyser cost. Figure S3 and Figure S4 show a graphic representation of the economic models for the estimation of production costs of fossil and green ammonia and methanol, respectively. The data used is presented in the following sections.

### Energy and materials inputs and outputs

The mass and energy balances for both fossil and green ammonia are taken from D’Angelo et al.1 and shown in Table S1. The inventories for fossil and green methanol are similarly retrieved from González-Garay et al.3 and are shown in Table S2.

Table S1. Inputs and outputs of the ammonia production routes. These values are extracted from the work done by D’Angelo et al.1.

| Material/energy stream | Fossil route | Green route |
| --- | --- | --- |
| Feedstock |  |  |  |
| Natural gas [kg] | 0.43 | - |
| Process water [kg] | 0.83 | - |
| Nitrogen [kg] | - | 0.82 |
| Hydrogen [kg] | - | 0.18 |
| Utilities |  |  |  |
| Heating – natural gas [kg] | 0.10 | - |
| Electricity [kWh] | 935.39 | 469.89 |
| Cooling water [GJ] | 18.12 | 3.12 |
| CO2 emissions [kg] | 1.43 | - |

Table S2. Inputs and outputs of the methanol production routes. These values are extracted from the work done by González-Garay et al.3.

| Material/energy stream | Fossil route | Green route |
| --- | --- | --- |
| Feedstock |  |  |
| Natural gas [kg] | 0.51 | - |
| Process water [kg] | 0.85 | - |
| CO2 feedstock [kg] | - | 1.45 |
| Hydrogen [kg] | - | 0.19 |
| Utilities |  |  |
| Heating – natural gas [kg] | 0.15 | - |
| Electricity [kWh] | 74 | 300 |
| Cooling water [GJ] | - | 4.95 |

### Data for CAPEX calculations

The CAPEX contributions for the case of ammonia are extracted from the work done by D’Angelo et al.1. With 2019 as the reference year, the contribution of CAPEX is estimated to be around 133 USD t–1 and 80 USD t–1 of ammonia for the fossil and green routes, respectively. Similarly, for the case of methanol, the CAPEX is extracted from the work done by González-Garay et al.3. For 2019, the CAPEX contribution stood at around 56 USD t–1 and 92 USD t–1 of methanol for the fossil and green routes, respectively.  All costs have been converted to USD2020-22 using the CEPCI (Table S3) while considering different operating years other than 2019.

Table S3. Chemical Engineering Plant Cost Index (CEPCI) from 2019 to 2022. Due to lack of availability of data, an average from January 2022 to September 2022 is considered as the CEPCI for 2022.

| Year | CEPCI |
| --- | --- |
| 2019 | 607.5 |
| 2020 | 596.2 |
| 2021 | 708.0 |
| 2022 (average till September) | 817.5 |

### Data for OPEX calculations

The prices of feedstock and utilities other than natural gas, electricity, CO2 feedstock, hydrogen are based on D’Angelo et al.1 (Table S4). These prices are adjusted for inflation using the CEPCI when considering data for a different year.

Table S4. Cost parameters used in the OPEX calculations. All costs are presented in USD2019. These costs are further converted to USD2020-22 using the CEPCI.

| Flow |  | Value |
| --- | --- | --- |
| Ammonia |  |  |
| Water [USD t–1] |  | 0.18 |
| Cooling water [USD GJ–1] |  | 0.38 |
| CO2 emissions [USD t–1] |  | 30 |
| Nitrogen [USD t–1] |  | 91 |
| Methanol |  |  |
| Steam [USD t–1] |  | 15.6 |
| Cooling water [USD m–3] |  | 0.03 |
| Catalyst [USD t–1] |  | 136 |
| Wastewater treatment [USD m–3] |  | 1.64 |

For natural gas, we consider the monthly average spot prices at the Title Transfer Facility (TTF) hub in the Netherlands as reported in the Commodity Market Outlook of the World Bank (see Figure 1 in the main manuscript)6. The Dutch TTF hub has gained significant importance due to its high trade volumes, and currently acts as an European and global benchmark for natural gas prices7. Moreover, we consider monthly average electricity prices as shown in Figure S58.

The cost of CO2 from Direct Air Capture (DAC) is taken from Keith et al.5  and updated with the new natural gas and electricity prices, thus obtaining the monthly cost shown in Figure S6. It should be noted that the price of CO2 capture is highly influenced by the natural gas and electricity prices as around 0.1 kg of natural gas and 77 kWh of electricity is required per kg of CO2 captured.

The levelised cost of hydrogen (LCOH) is calculated as in D’Angelo et al.1. A Proton Exchange Membrane (PEM) electrolyser with a stack lifetime of 60,000 hours is considered for the analysis1. The parameters used to calculate the LCOH are shown in Table S5.

Table S5. Parameters for the Proton Exchange Membrane (PEM) electrolyser. All values are in reference to the year 2019. These values are used to calculate the levelised cost of hydrogen using the methodology explained further.

| Component | Value |
| --- | --- |
| Nominal input power [kW] | 5,000 |
| Nominal hydrogen flow [kg hr–1] | 90 |
| Stack lifetime [hr] | 60,000 |
| Power demand electrolyser [kWh Nm3H2–1] | 5 |
| Capital cost [USD kW–1] | 1,124 |
| Interest rate [%] | 6.4 |
| Other costs factor (fraction of initial CAPEX) | 0.04 |
| Decommissioning factor (fraction of depreciable capital) | 0.10 |
| Equipment replacement factor (fraction of initial CAPEX) | 0.15 |
| Construction period [yr] | 2 |
| Start-up period [yr] | 1 |
| Start-capacity [%] | 50 |
| Annual operation [hr] | 8,760 |

First, we calculate the life span of the PEM electrolyser using the stack lifetime and the capacity factors for the specific technology (on-shore wind or solar PV) from Table S6 (Equation S1).

|  | (Equation S1) |
| --- | --- |

The amount of hydrogen produced annually is then calculated assuming a nominal hydrogen flow per stack using Equation S2.

|  | (Equation S2) |
| --- | --- |

Assuming a start capacity of 50%, the CAPEX is estimated using the capital cost and nominal input power employing Equation S3. Notably, the capital cost is with reference to the year 2019 and is updated for inflation using the CEPCI when considering data for a different year.

|  | (Equation S3) |
| --- | --- |

The amount of hydrogen produced throughout the life span of the electrolyser is then calculated using Equation S4. We assume that in the first year of operation the plant works at a 50% capacity. Therefore, only for the first year, the hydrogen produced is multiplied by a factor of 0.5.

|  | (Equation S4) |
| --- | --- |
| with H2 production capacity= 0.5 i = 11.0 i > 1 |  |

In the next step, the total OPEX are calculated by summing the electricity costs (Equation S5) using the levelised cost of electricity (LCOE), other costs as 4% of initial CAPEX (Equation S6), plant decommissioning as 10% of initial CAPEX (Equation S7), and equipment replacement costs as 15% of depreciable CAPEX.

|  | (Equation S5) |
| --- | --- |

|  | (Equation S6) |
| --- | --- |

|  | (Equation S7) |
| --- | --- |

|  | (Equation S8) |
| --- | --- |

|  | (Equation S9) |
| --- | --- |

We retrieved European average LCOE for on-shore wind and solar electricity and their corresponding capacity factors from the International Renewable Energy Agency (IRENA)9 for 2019 to 2021, as shown in Table S6.

Table S6. Levelised cost of electricity (LCOE) in USD MWh–1 and capacity factors (CF) in Europe.

| Power technology | 2019 |  |  |  | 2020 |  |  |  | 2021 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Low | High | CF | Avg. | Low | High | CF | Avg. | Low | High | CF |
| Wind - on shore | 67 | 37 | 96 | 35.6% | 45 | 35 | 65 | 36.0% | 42 | 26 | 59 | 39.0% |
| Solar PV | 68 | 52 | 190 | 17.5% | 57 | 39 | 163 | 16.1% | 48 | 29 | 120 | 17.2% |

In the final step, the LCOH is calculated by summing the CAPEX and OPEX and dividing by the total amount of hydrogen produced as shown in Equation S10.

|  | (Equation S10) |
| --- | --- |

Due to data gaps for 2022, we determine first the LCOH for 2021 and then adjust it for inflation using the CEPCI. The calculated average, low, and high LCOH for each power technology in each year is reported in Table S7.

Table S7. Levelised cost of hydrogen (LCOH) in USD kg–1. Section S1.3 provides a comprehensive explanation of the assumptions and methodology employed for computing the LCOH values.

| Power technology | 2019 |  |  | 2020 |  |  | 2021 |  |  | 2022 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Low | High | Avg. | Low | High | Avg. | Low | High | Avg. | Low | High |
| Wind - on shore | 6.88 | 5.23 | 8.48 | 5.64 | 5.09 | 6.74 | 5.38 | 4.50 | 6.31 | 6.21 | 5.19 | 7.29 |
| Solar PV | 9.22 | 8.34 | 15.9 | 9.05 | 8.06 | 14.88 | 8.21 | 7.17 | 12.17 | 9.48 | 8.28 | 14.06 |

## Carbon footprint assessment

The activities used from the Ecoinvent v3.510 database to calculate the carbon footprint are shown in Table S8.

Table S8. Inventories used from Ecoinvent v3.5 database10 for the background system. These activities are used to calculate the global warming potential and the avoidance costs for the different routes of production of ammonia and methanol.

| Flow | Output | Activity in Ecoinvent v3.5 |
| --- | --- | --- |
| Natural gas | Fossil ammonia and methanol, CO2 from DAC, H2 via SMR + CCS | natural gas production [RoW] |
| Process water | Fossil ammonia and methanol, CO2 from DAC | tap water production, conventional treatment [RoW] |
| Grid electricity | All scenarios | market group for electricity, high voltage [GLO] |
| Air separation facility | Nitrogen | air separation facility construction [RoW] |
| Cooling water | All scenarios | water, cooling, unspecified natural origin [RoW] |
| Solar electricity for water electrolysis | H2 via solar | electricity production, photovoltaic, 570kWp open ground installation, multi-Si [RoW] |
| Wind electricity for water electrolysis | H2 via wind | electricity production, wind, >3MW turbine, onshore [RoW] |
| Calcium carbonate for DAC technology | CO2 from DAC | market for calcium carbonate, precipitated [RER] |
| Heat from steam | Non-fossil methanol | market for heat, from steam, in chemical industry [RER] |
| Wastewater treatment | Non-fossil methanol | market for wastewater, average [RoW] |

## Model validation

The production cost estimates for fossil ammonia and methanol are validated with their respective market prices using the Pearson correlation coefficient (r). The production cost estimates for ammonia calculated via techno-economic models were compared with the data extracted from Green Markets® (Figure S7). Green Markets® has recently published the market price of fossil ammonia from July 2019 to July 2022 for Western Europe. Figure S7a show the evolution of Europe's market price and production cost (estimated in this work). In Figure S7b, we compare the two data series using the Pearson correlation coefficient (r). The r value is found to be around 0.97 showing strong correlation between the two data series.

Market prices for European methanol were retrieved from Methanex Corporation (January 2015 to November 2022). Similar to ammonia, the data from Methanex is used to validate our production cost estimates from January 2015 to November 2022. The evolution of Europe’s market price and estimated production cost is depicted in Figure S8a. The r value from January 2015 to July 2021 is calculated to be around 0.75 showing a sufficient correlation between the two data series. When the r value is calculated for the entire time range (January 2015 to November 2022), the coefficient decreases to 0.68. Also, there is a visible difference between the market price and estimated production cost for fossil methanol from July 2021. This difference is thought to exist due to several reasons. On the one hand, Methanex has production sites in countries not affected by the current volatility of natural gas prices and hence produce methanol at a lower cost. When this methanol is exported to Europe, the market price is expected to be lower than the cost of production in Europe. On the other hand, we use monthly average natural gas spot prices from the TTF trading hub in the Netherlands. Companies with ongoing long-term contracts may secure a more favourable gas price compared to the current spot price, and the production cost would be lower as well. However, we would expect that the price of natural gas agreed upon in deals closed since the outbreak of the energy crisis would depend on the natural gas TTF index. Therefore, our estimates do not correspond to actual market values, but to the minimum selling prices in the market if those chemicals were produced in EU at the corresponding natural gas prices.

## Additional results

For the sake of clarity in graphical depictions, some additional results were not included in the main manuscript. Figure S9 shows the monthly average natural gas spot prices in addition to the fossil ammonia and methanol production cost. Calculations were performed considering the effect of inflation and the monthly fluctuations in natural gas and electricity prices (the evolution of electricity prices in Europe is displayed in Figure S5). Notably, ammonia and methanol production costs have increased steadily since 2020, due to the rising inflation as a consequence of the COVID-19 pandemic, with a sharp increase since February 2022, after the Russian invasion of Ukraine.

The production cost for both fossil and green routes of ammonia and methanol production throughout the entire period of discussion is presented in Figure S10.

Figure S11 illustrates the cost breakdown for the production of ammonia and methanol through fossil and green routes. Of all the components, hydrogen makes the greatest contribution to the average production cost of green chemicals.

Figure S12 shows the avoidance cost for the different routes of ammonia and methanol production including the lower and upper bounds of the hydrogen costs from Table S7.

### Figures

Figure S1. Flowsheet for the production of fossil ammonia. For green ammonia, green hydrogen from water electrolysis and nitrogen from cryogenic air separation are mixed in the ratio 3:1 to prepare the Haber-Bosch feed1.

Figure S2. Flowsheet for the production of fossil methanol. For green methanol, CO2 from DAC and hydrogen from water electrolysis are mixed in the ratio of around 1:3 and the reactor operates at 50 bar3.

Figure S3. Techno-economic model for the estimation of production costs for (a) fossil ammonia and (b) green ammonia. Other utilities include cooling water and wastewater treatment.

Figure S4. Techno-economic model for the estimation of production costs for (a) fossil methanol and (b) green methanol. Other utilities include cooling water and wastewater treatment.

Figure S5. Evolution of monthly average electricity prices in Europe. Notably, electricity costs per kWh increased steadily since July 2021 following similar reasons as discussed in the case of natural gas (see Figure 1 in the main manuscript). The monthly average prices of electricity along with natural gas are used in the techno-economic models for calculating the production cost estimates.

Figure S6. Evolution of carbon dioxide prices using DAC technology. Carbon dioxide capture prices are highly volatile and dependent on the price of natural gas and electricity. Error bars display the capture cost considering the different Capital Recovery Factors (CRF) from the work done by Keith et al.5.

Figure S7. Ammonia model validation. (a) shows the time evolution of the production cost estimates in comparison to the market price extracted from Green Markets®. (b) shows the correlation between the market price and the production cost estimate. A Pearson correlation coefficient (r) value of 0.97 is estimated showing a strong correlation between the two data series.

Figure S8. Methanol model validation. (a) shows the time evolution of the production cost estimates in comparison to the market price extracted from Methanex. (b) shows the correlation between the market price and the production cost estimate. A Pearson correlation coefficient (r) value of 0.75 is estimated for the time period January 2015 to July 2021 (shown) and 0.68 for the time period January 2015 to November 2022 (not shown).

Figure S9. Natural gas spot prices and production cost for fossil ammonia and methanol in Europe. (a) Fossil ammonia produced via the Haber-Bosch process with grey hydrogen from steam methane reforming (SMR) using natural gas as feedstock. (b) Fossil methanol synthesised from natural gas.

Figure S10. Production cost for green ammonia and methanol in Europe compared with the fossil counterpart in USD kg–1. Green ammonia produced via the Haber-Bosch process with (a) blue hydrogen from steam methane reforming (SMR) coupled with carbon capture and storage (CCS) and green hydrogen from water electrolysis using (b) solar photovoltaic (PV) electricity or (c) on-shore wind electricity. Methanol synthesised from captured CO2 and (d) blue hydrogen –SMR coupled with CCS–, (e) PV-based electrolytic hydrogen or (f) wind-based electrolytic hydrogen. The error bands indicate the lower and upper bounds for the levelised cost of hydrogen (see Table S7).

Figure S11. Breakdown of production costs for fossil and green ammonia (a) and methanol (b). Error bars display the production cost considering the lower and upper bound for the levelised cost of hydrogen.

Figure S12. Avoidance costs for green (a) ammonia and (b) methanol production routes in USD kg–1 CO2-eq. Error bars display the production cost considering the lower and upper bound for the levelised cost of hydrogen (Table S7).

## References

1	S. C. D’Angelo, S. Cobo, V. Tulus, A. Nabera, A. J. Martín, J. Pérez-Ramírez and G. Guillén-Gosálbez, ACS Sustain. Chem. Eng., 2021, 9, 9740–9749.

2	M. Appl, in Ullmann’s Encyclopedia of Industrial Chemistry, ed. Wiley-VCH Verlag GmbH & Co. KGaA, Wiley-VCH Verlag GmbH & Co. KGaA, Weinheim, Germany, 2011.

3	A. González-Garay, M. S. Frei, A. Al-Qahtani, C. Mondelli, G. Guillén-Gosálbez and J. Pérez-Ramírez, Energy Environ. Sci., 2019, 12, 3425–3436.

4	K. M. V. Bussche and G. F. Froment, Methanol kinetics, 1996, vol. 161.

5	D. W. Keith, G. Holmes, D. St. Angelo and K. Heidel, Joule, 2018, 2, 1573–1594.

6	The World Bank, Commodity Markets Outlook: The Impact of the War in Ukraine on Commodity Markets, The World Bank Group, Washington DC, 2022.

7	P. Heather, European Traded Gas Hubs: the supremacy of TTF, The Oxford Institute for Energy Studies, 2022.

8	EMBER, European power price tracker, https://ember-climate.org/data/data-tools/europe-power-prices/, (accessed February 23, 2023).

9	IRENA, Renewable power generation costs in 2021, International Renewable Energy Agency, 2021.

10	G. Wernet, C. Bauer, B. Steubing, J. Reinhard, E. Moreno-Ruiz and B. Weidema, Int. J. Life Cycle Assess., 2016, 21, 1218–1230.