Response to Reviewers for GC-PER-03-2023-001053

Comments in blue, replies in black, and actions in bold. Page and line numbers refer to the main article with highlighted changes.

Reviewer #1

1.0. The manuscript is a timely piece to urge for the revolutionary transition to green and sustainable practice in chemical industry. The manuscript is well written in general and should attract wide readership. The authors analysed the costs of several green routes for producing green ammonia and methanol, in comparison with the fossil-based practice. The discussion is comprehensive and, yet, it focuses on a bigger picture of the driving forces and R&D directions rather than discussion of key findings from the cost analysis. For stronger linkage with “Results”, I suggest starting the Discussion with a new short paragraph, highlighting the key determinants of green chemical production cost that were revealed by the analysis. More specific comments are suggested below for consideration.

We thank the Reviewer for highlighting our work’s urgency and impact and for their insightful comments. Following the Reviewer’s suggestion, a short paragraph highlighting the key results from our analysis is now included at the beginning of the discussions section (page 6, line 23, right column).

In addition to addressing the specific comments of the Reviewer below, we have included supplementary assessments to further enhance the granularity and scope of the article. We performed regionalised production cost assessments, both within and outside Europe. Figures S9 and S10 in the Electronic Supplementary Information (ESI) show the regional cost estimates for ammonia and methanol in several European countries, including Germany, France, Spain, Italy, the Netherlands, and others. Furthermore, in Figure S11 of the ESI, we compare the production cost estimates between Europe and the United States (US). This assessment was conducted to understand if regions other than Europe are incentivised to transition towards the green routes for ammonia and methanol production. We have included additional text in the main manuscript to further elaborate on the main findings of these assessments (page 3, line 17, left column).

  - “…biomass routes could also be deployed, yet they may shift environmental burdens to other categories despite their economic appeal.” The authors may slightly elaborate.

Previous studies have shown that production pathways based on biomass are economically competitive with their fossil counterparts and have lower climate change impacts. However, the economic appeal of these routes must be considered in conjunction with all the environmental impacts throughout their life cycle, such as land use change, resource consumption (including land, water, and energy), and potential impacts on biodiversity and ecosystem services. While hydrogen from biomass and BECCS routes could significantly reduce climate change and ocean acidification impacts, they may also increase the burden on biodiversity and nitrogen flows [Galán-Martín et al., One Earth, 2021, 4, 565–583]. The original text has been modified to better profile this discussion (page 2, line 22, right column).

1.2. Figure 2: The authors may consider adopting a stacked line chart as in Figure 1 because readers could be interested in the visual comparison among the green routes.

We appreciate the Reviewer’s suggestion regarding the appearance of Figure 2 in the main manuscript. We have addressed the Reviewer’s comment by adding a new figure (Figure S12) in the ESI. This item presents the production cost of fossil and green routes stacked in one plot for ammonia and methanol. Further, we have included supplementary text in the caption of Figure 2 of the main manuscript to reference the corresponding figure in the ESI.

1.3. “…the average European levelised cost of wind‐based electrolytic hydrogen steadily decreased…” In Figure 3, the H2 cost slightly fluctuates instead of showing a decreasing trend. Please clarify in the text.

Although the average European levelised cost of wind-based electrolytic hydrogen had steadily decreased from 2019 until 2021 (as shown in Figure 3), we observed a price increase in 2022. This increase was due to the methodology used to calculate the levelised cost of hydrogen. Since there was a lack of available data for the levelised cost of electricity in 2022, we determined the levelised cost of hydrogen for 2021 and adjusted it for inflation using the CEPCI. A more detailed methodology is provided in Section 2.4 of the ESI. In order to ensure a more comprehensive discussion on this aspect, we have included these considerations in the revised text (page 6, line 5, left column).

1.4. “… the cost of on‐shore wind electricity is expected to reach 40 USD MWh−1 by 2030. This is expected to result in a decrease in the hydrogen’s levelised cost to approximately 2.7 USD kg−1...” To present a more forward-looking vision, the authors may consider adding a scenario in Figure 2 assuming such cost reduction, and see how it compares with the fossil-based production.

We agree with the Reviewer that cost projections would help evaluate the feasibility of green routes for ammonia and methanol production. However, due to the unpredictability of natural gas prices, it is difficult to accurately project the production cost of fossil routes and compare it with that of green routes. To address the issue of unpredictability, we conducted a sensitivity analysis using varying natural gas and electricity prices. This study enabled us to calculate the breakeven price of natural gas, at which green routes will become economically feasible. We have also incorporated the future cost estimates of wind-based routes in Figure S16 to offer a more forward-looking perspective. The levelised cost of hydrogen is projected to decrease to 2.8 USD kg−1 by 2030 and further decline to 2.1 USD kg–1 by 2050, as described in [Brändle et al., Appl. Energy, 2021, 302, 117481]. According to our future estimates, wind-based ammonia will cost between 0.7 and 0.9 USD kg–1, while wind-based methanol is projected to cost between 0.8 to 1.0 USD kg–1. Additionally, the breakeven price of natural gas is calculated to be around 
0.8–1.1 USD kg–1 for ammonia and 1.0–1.4 USD kg–1 for methanol, respectively. We found that the natural gas prices in 2022 surpassed these breakeven prices, strengthening our work’s conclusions. Furthermore, we have added a new paragraph in the results section to discuss these projections (page 6, line 28, left column).

Reviewer #2

2.0. The article focuses on reducing CO2 emissions from chemical industries that produce two major platform chemicals. The work highlights the, green ammonia, production through wind‐powered water electrolysis and methanol production through CO2 capture and wind‐based water electrolysis as an economically appealing alternative. However, the manuscript was vaguely written without proper discussions and justifications in terms of CO2 reduction. The authors have not appropriately presented the LCA and economic analysis outcomes. Comparative study of various technologies with use of alternative energy (methanol and ammonia) process were already reported in the literature. Although, the authors have projected the several economic analyses, the results of those analyses were not appropriate. The cost calculations may be viable only with respect to specific region/chosen country and adopted process. It may not be valid if there is change in process, change in market value or change in location due to the availability of raw materials, energy, technology etc. Due to lack of novelty, poor methodology planning and representation of results, this work cannot be accepted for publication in the Green chemistry journal.

We appreciate the constructive criticism from the Reviewer; however, we regret that the novelty and significance of our results went unnoticed. We acknowledge that numerous studies exist in the literature on techno-economic and life cycle assessments for various ammonia and methanol production routes. However, this perspective pioneers a demonstration on how the escalating energy prices could drive Europe to lead the grand transition towards adopting sustainable production pathways if proven economically viable. To the best of the authors’ knowledge, this type of analysis is currently absent in the literature, and our work aims to fill this gap. In parallel, we revised our manuscript according to the Reviewer’s suggestions to increase the depth and scope of our manuscript.

The Reviewer recommended a broader analysis to assess the viability of production costs in different regions within and outside Europe. With this regard, we implement regionalisation of our production cost estimates in our revised work, as explained in response to comments 2.9 and 2.12. This analysis provides more detailed insights and reinforces our main findings, highlighting Europe’s potential to lead the grand transition towards sustainable practices in the chemical industry. In addition to the regional assessments, a forward-looking perspective is explained in response to comment 2.10. The main conclusion of this study is that the natural gas prices in 2022 were much higher than the breakeven prices calculated for future estimates, thereby reinforcing our discussions.

2.1. In introduction, avoid citing many references (ref 10-25) at for a single statement. Modify it by providing appropriate citation.

We have revised the introduction by providing appropriate citations for the statement in question (page 2, line 3, left column).

2.2. In introduction, explain the various process so far developed along with SMR for ammonia production.

The original introduction explaining the conventional Haber-Bosch process utilising natural gas and the green Haber-Bosch process employing hydrogen generated from water electrolysis for ammonia synthesis has been expanded (page 1, line 28, left column). We address another viable route for hydrogen production required by the Haber-Bosch process, which involves utilising renewable feedstocks like biomass. Biomass can be converted to syngas through gasification, which can then be further processed to produce hydrogen. Furthermore, a promising long-term option for sustainable ammonia production would be the direct electrocatalytic reduction of nitrogen. This pathway is highly appealing due to its association with renewable electricity sources. However, the current practicality of this technology at an industrial scale is limited. Therefore, we have expanded this section to incorporate these routes (page 1, line 5, right column).

2.3. In introduction, applications of methanol and production routes needs to be elaborated.

Methanol finds its immediate application as a fuel or fuel additive, offering cleaner combustion and reduced emissions compared to fossil fuels. Furthermore, its versatility allows it to serve as an intermediate in the production of olefins and aromatics. Methanol is also extensively utilised to manufacture diverse compounds, such as formic acid, dimethyl ether, and methylamine. We have modified the introduction accordingly to provide a broader context regarding methanol applications (page 1, line 15, left column). Regarding the existing methanol production routes, we would like to highlight the text in the original introduction, where the conventional methanol production employing hydrogenation of CO is described in detail. Sustainable methanol production utilising captured CO2 and low-carbon hydrogen produced through water electrolysis is likewise elaborated (page 1, line 17, right column). As explained in response to comment 2.3, another sustainable approach is to use biomass as a feedstock for methanol production. An upcoming technology is the co-electrolysis of CO2 and water using solid oxide electrolysers at high temperatures. Due to their reliance on renewable power sources, this pathway is a promising alternative for producing syngas required for methanol synthesis. We have restructured the introduction to provide a broader context on methanol production routes (page 1, line 25, right column).

2.4. In introduction, explain the various green processes developed so far by using renewable energy/route for ammonia and methanol synthesis.

As already discussed in the responses to the Reviewer’s comments 2.2 and 2.3, the introduction has been restructured and modified to include various production routes (green and fossil) already developed for ammonia and methanol (page 1, line 28, left column). In addition, we would like to highlight that a paragraph devoted to discussing upcoming technologies for the green process was already incorporated in the discussions section of the manuscript (page 7, line 7, left column). This paragraph discusses the low TRL technologies currently in development for green ammonia and methanol synthesis. It also highlights the disadvantages and the scope of improvement for each production route. The discussion section has been restructured and reformatted following the Reviewer’s comments 1.0 and 2.13.

2.5. Provide the futuristic projections (example: 2025, 2030, 2050) for the demand, CAGR and production of ammonia and methanol. Also compare the projections with GHG emissions and green production process.

We appreciate the Reviewer’s suggestion to include futuristic projections for the demand, compound annual growth rate (CAGR), and ammonia and methanol production. The demand for ammonia is directly related to the growing population. Around 185 Mt yr–1 of ammonia is produced globally, which is responsible for GHG emissions of approximately 0.5 Gt CO2-eq yr–1 [Ammonia Technology Roadmap: Towards more sustainable nitrogen fertiliser production, International Energy Agency, 2021]. The demand for ammonia is expected to increase to 355 Mt yr–1 by 2050 [Ammonia Technology Roadmap: Towards more sustainable nitrogen fertiliser production, International Energy Agency, 2021]. Similarly, global methanol production currently stands at 100 Mt yr–1, corresponding to GHG emissions of around 0.3 Gt CO2-eq yr–1 [Innovation Outlook: Renewable Methanol, International Renewable Energy Agency, 2021]. According to future projections, methanol demand is predicted to increase to 500 Mt yr–1 by 2050 [Global Renewables Outlook: Energy transformation 2050, International Renewable Energy Agency, 2020]. The corresponding text was added to the introduction to highlight the global demand and importance of ammonia and methanol (page 1, line 9, left column).

2.6. In results, how can you say that increase in natural gas prices leads to search for alternative fuels? GHG emissions, depletion of fossil reserves, need and demand of chemicals leads in search for alternative low carbon chemicals.

While we agree with the Reviewer that factors such as GHG emissions, the depletion of fossil reserves, and the demand for chemicals contribute to the search for alternative low-carbon chemicals, we also believe that it is crucial to acknowledge the significant role that energy prices can play in the transition towards sustainable production pathways. Notably, natural gas reserves are abundant and will continue to provide a reliable energy source for many years. However, the novelty of this work is that the surging energy prices (natural gas and electricity) can act as a catalyst and drive Europe to lead the grand transition towards sustainable production pathways. The escalation in energy prices has heavily impacted various industries that rely on natural gas. Therefore, in the face of high energy prices, the industry will be compelled to seek alternatives that are not only economically viable but also environmentally sustainable. To enhance the clarity of our approach, the results sub-section heading was changed to: “Surging energy prices drive low-carbon technologies.”

2.7. In results, discuss the Fig S1 and S2 in the running text and place the figure in the main manuscript.

We believe that the placement of Figure S1 and Figure S2 is appropriate. The reason is that the primary goal of this work is to convey the message of how green routes fare economically in the current geopolitical scenario. Section 1 of the ESI, along with Figures S1 and S2, contains extensive information about the conventional ammonia and methanol production process, which has already been discussed in numerous studies. Placing these figures in the main manuscript could potentially dilute the main message, thereby distracting from the main findings and key points discussed. However, we agree that there could be better referencing to these figures in the running text. In attention to the Reviewer’s comment, we have further enhanced the referencing and contextualisation (page 2, line 33, left column).

2.8. Focus can also be given to reduction of GHG emissions by green synthesis instead of economic aspects.

We appreciate the Reviewer’s suggestion to also focus on reducing GHG emissions instead of solely the economic aspects. As described in the main manuscript, the green routes for producing ammonia and methanol have consistently been found to be more environmentally sustainable than their fossil counterparts. However, the bottleneck has always been the economics of these pathways. Various studies conducted in the literature have concluded that these routes are economically infeasible. Consequently, we have focused our case study solely on economics, demonstrating how rising energy prices could present a win-win scenario for Europe if we transition to more sustainable pathways for ammonia and methanol production. However, we acknowledge the convenience of highlighting the reduction in GHG emissions alongside the economic aspects. Therefore, we have calculated the avoidance costs for ammonia and methanol, which integrate the economics and the global warming potential (GWP) associated with each production route. Therefore, an additional paragraph underscoring the estimated GWP values and the significant environmental advantages of green ammonia and methanol compared to their fossil-based counterparts is incorporated in the revised manuscript (page 6, line 48, left column).

2.9. The cost calculations may be viable only with respect to specific region/chosen country and adopted process. It may not be valid if there is change in process, change in market value or change in location due to the availability of raw materials, energy, technology etc.

We thank the Reviewer for highlighting this aspect. Our analysis used average values for Europe. However, we acknowledge that these values may differ regionally. We included a regional assessment for ammonia and methanol within Europe (see Figure S9 and Figure S10 for ammonia and methanol, respectively) and outside Europe (see comment 2.12, Figure S11). At the peak of natural gas prices, the production costs of fossil ammonia in Europe ranged from 2.1 to 2.5 USD kg–1. Spain achieved the lowest production cost among European countries, while Denmark had the highest. Similarly, the production costs for fossil methanol ranged from 2.1–2.2 USD kg–1. In 2022, France recorded the highest production costs for ammonia and methanol via the wind pathway at 1.6 USD kg–1 and 2.2 USD kg–1, respectively, while the Netherlands had the highest production costs for the solar pathway at 2.5 USD kg–1 for ammonia and 3.0 USD kg–1 for methanol. In contrast, Spain demonstrated the lowest production costs for wind-based ammonia at 1.2 USD kg–1 and solar-based ammonia at 2.0 USD kg–1. Moreover, Spain exhibited the most cost-effective wind and solar-based methanol production at 1.7 and 2.6 USD kg–1, respectively. An additional paragraph emphasising the key results from this analysis is included in the revised manuscript (page 3, line 17, left column). In addition to Figures S9 and S10, Tables S9 and S10 were included in the ESI, presenting the regional data for the levelised cost of electricity and hydrogen for solar and wind, respectively. Furthermore, Section 5 of the ESI was revised to incorporate the methodology used to calculate the regional costs.

2.10. How can the calculated economic for ammonia and methanol will be same (next 10, 20, 50 years etc) in the future?

We agree with the Reviewer that the calculated economic values for fossil ammonia and methanol and green ammonia and methanol may or may not be similar in the next 10, 20, or 50 years. However, due to the unpredictability of energy prices, projections of the production cost of fossil routes are inaccurate and cannot be compared with those of the green routes. In light of this uncertainty, we perform a sensitivity analysis by considering varying energy prices to estimate the production costs of the fossil routes and compare them with the future wind-based ammonia and methanol costs. This analysis helps us define the breakeven price of natural gas, at which green routes will become economically feasible. We have incorporated these future estimates in Figure S16 to provide a more insightful outlook. By 2030, the levelised cost of hydrogen is projected to decrease to 2.8 USD kg−1 and further decline to 2.1 USD kg–1 by 2050, according to [Brändle et al., Appl. Energy, 2021, 302, 117481]. According to our estimates, production costs for wind-based ammonia will be between 0.7 and 0.9 USD kg–1, while wind-based methanol ranges from 0.8 to 1.0 USD kg–1. The breakeven price of natural gas is calculated to be around 0.8–1.1 USD kg–1 for ammonia and 1.0–1.4 USD kg–1 for methanol, respectively. These breakeven prices are significantly lower than the natural gas prices in 2022, further reinforcing our conclusions. We have included this analysis in the results section (page 6, line 28, left column).

2.11. Highlight the recent advances and anticipated technologies in near future for the sustainable processes for ammonia and methanol production.

Referring to the Reviewer’s comments 2.2, 2.3, and 2.4, the introduction of the revised manuscript now incorporates a more detailed discussion on existing and upcoming technologies regarding both ammonia and methanol production (page 1, line 28, left column). We would also like to take this opportunity to shed light on the paragraph included in the discussions section that delves deeper into these upcoming technologies (page 7, line 7, left column). Notably, the potential of ammonia leaves, i.e., small electrocatalytic reactors coupled to solar power for direct fertiliser production at croplands, are emphasised. We also examine emerging catalysts for methanol synthesis, such as promoted indium oxide and zinc-zirconium oxide. We also elaborate on the scope of direct electrocatalytic conversion of CO2 using molybdenum‑based catalysts. Moreover, the discussion section also covers the current applicability of these technologies and identifies potential areas of improvement.

2.12. The authors have conducted a monthly techno‐economic assessment of Europe and the production costs for green, fossil ammonia and methanol. Nevertheless, in order to ascertain their economic competitiveness, it is necessary to compare them with other nations.

We appreciate the suggestion from the Reviewer and acknowledge that conducting this comparison would be a valuable study. Hence, we carried out similar calculations as those undertaken for Europe, but this time focusing on the United States (US). We consider this assessment of high interest since the US mainly remained unaffected by the energy crisis experienced in Europe. The comparison between production costs in Europe and the US is illustrated in Figure S11. Section 5.2 of the ESI elaborates on the methodology employed to estimate these production costs. In the US, the prices of fossil ammonia reached peak values of 0.8 USD kg–1, nearly 50% lower than the corresponding cost in Europe. Similarly, the highest production cost of methanol through the fossil route was 0.4 USD kg–1, which is 82% less compared to Europe. These variations can be attributed to the comparatively lower prices of natural gas and electricity in the US than Europe. Consequently, despite their environmental benefits, the green production routes in the US face significant economic burdens and remain non-competitive. The main manuscript has been updated to include the key results and references to corresponding figures in the ESI (page 3, line 16, right column).

2.13. Conclusion were vague and not effective, it needs a thorough revision.

Following the Reviewer’s suggestion, the discussions section was revised (page 7, line 7, left column). We are confident that our revised discussions section is now clearer and more concise. In the first paragraph, we summarise our study’s main findings, i.e., production costs of fossil ammonia and methanol in Europe increased due to surging energy prices, while green alternatives became economically viable in addition to being environmentally appealing. Notably, wind-based ammonia was more affordable than the fossil route for six months. Next, we highlight the energy crisis’s possible optimistic influence on Europe’s low-carbon chemicals industry and discuss the challenges of returning to low energy prices as experienced in 2019. Technological advancements and the positive effects of the learning curve will further help reduce the costs of green routes and make them even more appealing in comparison to the high volatility of fossil routes. In the third paragraph, we talk about Europe’s commitment to reducing GHG emissions and suggest that market-driven green ammonia and methanol production can help the European chemical industry reduce emissions at low costs. In paragraph four, concerning the Reviewer’s comments 2.11, we summarise upcoming sustainable ammonia and methanol production technologies. Given the proven volatility of fossil resources, we conclude by emphasising Europe’s advantage in leading the grand transition to sustainable, low-carbon chemical production.

2.14. Detailed methodology is lacking.

The detailed methodology can be found in the ESI, following editorial guidelines and considering that this article does not develop novel methods. We have also included a summary of the economic and avoidance cost calculations in the main manuscript. We incorporate a paragraph in the main manuscript more clearly informing the readers about the availability of the detailed methodology in the ESI (page 7, line 24, right column).

Reviewer #3

3.0. I think this is an interesting and timely article on green chemicals in Europe. The key hypothesis of this paper is that the price of fossil ammonia and fossil methanol has gone up in Europe because of the high price of natural gas and the price of green methanol and ammonia (made from green electricity) is now lower.

We express our gratitude to the Reviewer for recognising the timeliness and significance of our research and providing constructive feedback on our perspective. We include a point-by-point response to the specific comments of the Reviewer below.

In addition to these responses, we have incorporated complementing assessments in our revised manuscript. In one study, we perform a sensitivity analysis with varying natural gas and electricity prices to assess the volatility of energy prices. Further, future production cost estimates of wind-based routes are calculated, and the breakeven price of natural gas is evaluated such that the wind-based routes are economically appealing. These results are shown in Figure S16 of the ESI. The main conclusion of this study is that the natural gas prices in 2022 were much higher than the breakeven prices calculated for future estimates, thereby reinforcing our discussions. The revised manuscript contains a new paragraph underscoring these findings (page 3, line 17, left column). In another study, regionalised production cost assessments were conducted on a regional basis, both within and outside Europe. The ESI contains Figures S9 and S10, which display the regional cost estimates for ammonia and methanol in various European countries such as Germany, France, Spain, Italy, the Netherlands, and others. Further, Figure S11 in the ESI presents a comparison of production costs between Europe and the United States (US). The assessment aimed to determine if regions beyond Europe have incentives to transition towards sustainable production pathways for ammonia and methanol. These considerations have been included in the revised manuscript (page 3, line 16, right column).

3.1. The issue I have with this is that the price of electricity will also go up with the price of natural gas. The authors should show the price of electricity in Figure 1.

We agree that considering the cost of electricity is crucial when analysing the costs associated with fossil and green ammonia and methanol production routes. Therefore, we have modified Figure 1 in the main manuscript to include the price of electricity. However, it’s important to note that this price was already incorporated in our estimates of the production costs for fossil routes, as explained in the main manuscript and Section 2.1 of the ESI.

3.2. They should also comment on how these higher electricity prices will impact the price of green methanol and ammonia. The higher electricity prices will cause the price of H2 from solar or wind to increase. I recommend that this analysis be included and discussed in this paper.

Regarding the influence of grid electricity prices on the cost of green ammonia and methanol, our estimates for the green routes assumed the production of on‑shore wind and solar power within Europe. This renewable energy is then utilised for the production of green hydrogen. Therefore, the hydrogen production costs from water electrolysis are independent of the grid electricity prices and depend only on the levelised cost of on‑shore wind and solar power for the corresponding scenario, respectively. The grid electricity price is only used for the electricity required for the compressors during ammonia and methanol production. However, we acknowledge the valid argument that including the cost of green ammonia and methanol produced through hydrogen from grid-electricity-powered water electrolysis is a relevant comparison to consider. In light of these suggestions, we have included a new Figure S12 in the ESI, which illustrates the production costs of various pathways for ammonia and methanol production, including the use of grid electricity for the electrolyser. To calculate the levelised cost of grid-powered hydrogen, we use the levelised cost of electricity shown in Figure 1 of the main manuscript. From January 2019 to May 2021, the average production cost of ammonia and methanol from grid electricity is estimated to be 0.9 USD kg–1 and 1.2 USD kg–1, respectively. These costs are higher than the fossil routes (0.4 and 0.2 USD kg–1 for ammonia and methanol, respectively) but lower than the corresponding on-shore wind and solar pathways. However, starting from May 2021, as the grid electricity prices rise in parallel with the natural gas prices, the production cost of grid-based ammonia and methanol becomes economically unfavourable compared to the green routes. We have included a new paragraph in the revised manuscript to highlight these findings (page 3, line 1, left column).

Additionally, we acknowledge that the increasing natural gas prices will result in a higher levelised cost of on-shore wind and solar electricity. Therefore, it is essential to examine how these natural gas prices impact the levelised cost of hydrogen during the period of elevated natural gas prices in 2022. Due to a lack of data, the levelised cost of hydrogen for the year 2022 is calculated by using the levelised cost of hydrogen from 2021 and adjusting it for inflation using the CEPCI, as reported in Table S7. To determine the percentage share of natural gas prices in relation to the levelised cost of electricity, we extract the quantity of natural gas required to generate 1 kWh of on-shore wind and solar electricity from their respective Ecoinvent inventories (refer to Table S8 for the corresponding activity names of wind and solar electricity). Subsequently, we compare the levelised cost of hydrogen for 2022 under two scenarios: one where only inflation adjustments are made and another where both inflation adjustments and updated natural gas prices are considered. The findings are presented in Figure S18. Notably, both scenarios yield similar levelised costs of hydrogen. Hence, the assumption to use the inflation-adjusted values for the levelised cost of hydrogen proves to be robust.